# Evacuation Route Safety, Capacity, and Viability Analysis – Replication Methodology Specification

## Overview

This document summarizes the methodology used in the Berkeley AB‑747 evacuation analysis study and translates it into a reproducible technical specification suitable for software implementation.

The analysis evaluates roadway segments across four main dimensions:

1. Safety

2. Hazard exposure

3. Capacity

4. Viability

A fifth metric—Demographic Vulnerability—is calculated separately to understand social risk during evacuation.

The unit of analysis is the roadway segment derived from the city street centerline network.

## Primary Data Inputs

Road Network

• City street centerline dataset (polyline geometry)

• Lane counts

• Road classification

• Parking restrictions

• Segment geometry

Safety Data

• Vision Zero collision corridors (city dataset)

• Dead‑end road inventory

• Pavement Condition Index (PCI)

• International Roughness Index (FHWA)

• Bridge and tunnel inventory (BTS)

Hazard Data

• FEMA flood zones (100‑year / 500‑year)

• Landslide susceptibility (California Geological Survey)

• Earthquake hazard scenarios (USGS HayWired)

• Tsunami hazard zones

• Fire Hazard Severity Zones

Viability Data

• Gas stations (Google Places)

• Electric charging stations (PlugShare)

• Freeway ramps (centerline dataset)

Demand Data

• 2020 Census population

• Census population estimates

• LEHD employment data (OnTheMap)

• ACS vehicle ownership data

• UC Berkeley student population

• Highway Performance Monitoring System traffic counts

Demographic Vulnerability Data

• Poverty rate (ACS)

• Vehicle availability (ACS)

• Disability prevalence (ACS)

• Housing + transportation affordability index (Caltrans)

## Network Preparation

1. Import street centerline dataset.

2. Segment into individual roadway links.

3. Build a directed network graph.

Attributes stored for each segment:

segment_id

geometry

lane_count

speed_estimate

length

parking_left

parking_right

## Derived Geometry Metrics

Curvature

Compute radius of circumscribed circle using three sequential geometry points.

Segments with radius < 81 feet are considered high curvature risk.

Slope

Slope = elevation change / horizontal distance.

Categories:

0–2.3%

2.4–7%

7.1–15.2%

15.3–37.1%

Pavement Width

Effective width is reduced by parking:

• minus 8 ft if parking on one side

• minus 16 ft if parking both sides

Segments with travel width < 10 ft are considered constrained.

## Safety Score Calculation

Safety factors include:

• Collision corridor indicator

• Pavement width constraint

• Curvature threshold

• Slope category

• Pavement condition index

• Bridge or tunnel presence

Weighted safety score formula:

Safety Score =

collision +

width_constraint +

curvature +

(slope × 0.33) +

(pavement_quality × 0.33) +

(bridge_or_tunnel × 0.5)

Higher values indicate less safe roadway segments.

Classification thresholds are generated using Jenks Natural Breaks.

## Hazard Exposure

Hazard exposure factors are calculated but not included in the safety score.

Hazards considered:

• Flood zones

• Landslide susceptibility

• Earthquake intensity

• Tsunami exposure

• Fire hazard severity zone

All hazard metrics are normalized between 0 and 1.

## Roadway Capacity Calculation

Capacity is derived from the Highway Capacity Manual (HCM 2022).

Road types:

1. Two‑lane roads

2. Multilane roadways

3. Freeways

Two‑lane capacity examples:

5 mph → 225 vehicles/hr

20 mph → 900 vehicles/hr

40+ mph → 1700 vehicles/hr

Multilane roads:

1900 vehicles/hr per lane

Freeways:

2250 vehicles/hr per lane

Segment capacity = per‑lane capacity × lane count.

## Connectivity Score

Connectivity measures how many evacuation origin‑destination paths use each roadway segment.

Process:

1. Compute centroid for each census block group.

2. Identify exits leaving the city.

3. Compute shortest paths from each centroid to each exit.

4. Count how many OD paths traverse each segment.

Segments used by more paths receive higher connectivity scores.

## Accessibility Score

Accessibility measures the number of network connections.

Algorithm:

1. Create nodes at intersections.

2. Count links connected to each node.

3. Assign node connection counts to adjacent links.

Ramp connections receive a weight of +5.

Accessibility score = number of connections + ramp weights.

## Service Score

Service measures proximity to fuel and charging infrastructure.

Compute network travel time from each segment to:

• nearest gas station

• nearest electric charging station

Service score = travel_time_to_gas + travel_time_to_charger.

Higher scores indicate poorer access.

## Demand Modeling

Evacuation demand is estimated using population and employment data.

Steps:

1. Create ¼‑mile buffer around roadway segment.

2. Aggregate residents, employees, and students.

3. Convert population to vehicles using assumed occupancy.

Assumptions:

• Employee occupancy = 1 person per vehicle

• Students assumed to reside within city

• Tourists excluded

• Transit excluded

## Mobilization Curve

Vehicles enter the network gradually following an evacuation order.

Example peak mobilization:

Residents: 57% within 45–105 minutes

Employees and commuting students: 100% within first hour

Peak hourly demand is used for congestion analysis.

## Congestion Score

Congestion is computed using volume‑to‑capacity ratio.

v/c = demand / capacity

Converted to Level of Service (LOS):

A: <0.1

B: 0.1–0.2

C: 0.2–0.4

D: 0.4–0.6

E: 0.6–0.95

F: >0.95

## Demographic Vulnerability

Demographic vulnerability assesses population evacuation challenges.

Factors:

• Poverty rate

• Vehicle availability

• Disability prevalence

• Housing + transportation affordability

Each factor is normalized and combined.

Higher values indicate more vulnerable populations.

## Final Outputs

For each roadway segment the system outputs:

segment_id

capacity

safety_score

hazard_score

connectivity_score

accessibility_score

service_score

congestion_score_max

congestion_score_min

vulnerability_score

Values can also be aggregated by street name for reporting.

## Suggested Software Stack

Data processing

• Python

• GeoPandas

• PostGIS

Network analysis

• pgRouting

• NetworkX

• OSRM or Valhalla

Spatial classification

• mapclassify or scikit‑learn

Visualization

• Mapbox / Deck.gl

• QGIS

• Kepler.gl